# Wireless Network Needs and Requirements

Jesse Missaghian

University of Phoenix

CYB/360: Wireless Security

Jason Cupano

11/27/2023

Creating a wireless LAN-2-LAN network for International Plastics, Inc.'s corporate campus requires a nuanced approach that incorporates various technical, regulatory, and security considerations.

From a technical standpoint, my primary goal is to establish seamless connectivity that supports the dynamic nature of the business’ activities. This involves deploying a high-density wireless network with Access Points (APs) capable of handling a large numbers of simultaneous connections without degradation in service. The APs should support the latest Wi-Fi standards, such as 802.11ax (Wi-Fi 6), which offers increased data throughput and improved efficiency in highly demanding settings.

In terms of network design, I would support for a strong wireless mesh topology, which allows for flexible expansion and self-healing capabilities. According to Cisco’s Campus LAN and Wireless LAN Solution Design Guide, “Mesh networks are particularly beneficial in a campus setting as they provide redundancy, critical for maintaining connectivity if an AP goes down. The mesh design also facilitates easier scalability for future growth, allowing new buildings to be added to the network without extensive infrastructure overhauls” (2020). So a wireless mesh topology is an ideal choice for campus networks.

Adherence to Federal Communications Commission (FCC) regulations is non-negotiable. Compliance involves ensuring that all wireless equipment is FCC-certified, operating within the allocated frequency bands, and obeying the power output restrictions to prevent interference with other band users. Regular audits and firmware updates will be required to maintain compliance as FCC regulations evolve.

Cost-effectiveness will be accomplished through a combination of strategies. We'll use economies of scale by standardizing AP models across the campus, which can also simplify maintenance and management. Adopting cloud-managed wireless networking solutions can reduce the need for on-site network management hardware, cutting down capital expenditure.

Security is of paramount importance, especially considering the contractual obligations to clients. The network will implement WPA3-Enterprise for robust encryption and authentication protocols. Additionally, employing Protected Management Frames (PMF) will provide an added layer of protection against deauthentication attacks, which are common precursors to man-in-the-middle attacks.

The network infrastructure will be designed to incorporate security policies such as role-based access control, ensuring that users are granted the necessary permissions based on their job requirements and nothing more. This helps in minimizing the potential impact of any security breach by restricting lateral movement within the network.

A thorough risk assessment will inform the deployment of a security information and event management (SIEM) system, which will monitor the network for suspicious activities and automate responses to potential threats. This system will be pivotal in providing real-time security alerts and facilitating rapid incident response.

The proposed plan aims to create a wireless network that is robust, secure, and scalable, with careful consideration of technical specifications, regulatory compliance, cost management, and security. It is designed not only to meet the current needs of International Plastics, Inc. but to accommodate future technological advancements and business expansions.

| Frequency Band | Strengths | Weaknesses |
| --- | --- | --- |
| 2.4 GHz | - Excellent range and coverage due to longer wavelengths. - Better penetration through walls and solid objects, ideal for indoor coverage. - Broad device compatibility as most wireless devices support this band. | - Prone to interference from a myriad of common household devices like cordless phones, baby monitors, and Bluetooth devices. - Overcrowded due to a limited number of non-overlapping channels (only 1, 6, 11 in the USA). - Generally lower data throughput compared to higher bands. |
| 5 GHz | - Higher data rates and speed due to increased bandwidth availability. - Less crowded with more non-overlapping channels, leading to less interference. - Supports advanced features like channel bonding for wider channels and higher throughput. | - Limited range and weaker signal penetration through obstacles compared to 2.4 GHz. - Not all wireless devices are compatible with this band, although this is rapidly changing. - Can be more affected by attenuation, leading to reduced signal strength over distances. |
| 6 GHz (wi-Fi 6E) | - Latest technology providing even higher data rates and capacity. - Supports the newest Wi-Fi 6E standard, allowing for less congested airwaves. - More non-overlapping channels available, reducing interference and improving performance. | - Limited to devices that support Wi-Fi 6E, which are not as widespread yet. - Similar to 5 GHz in terms of range limitations and penetration issues. - May require more infrastructure investment to cover the same area as lower frequencies. |

References:

{sectiontitle} - campus LAN and wireless LAN Solution Design Guide. Cisco. (2020, May 4). https://www.cisco.com/c/en/us/td/docs/solutions/CVD/Campus/cisco-campus-lan-wlan-design-guide.html

Coleman, D. D., & Westcott, D. A. (2021). CWNA Certified wireless network administrator study guide: Exam CWNA-108 (6th ed.). John Wiley & Sons, Inc.